# Penalty Expert Report - Booster T1_23

# Purpose of the document

This report describes the penalty shot expert for the Booster T1 23-DoF robot, with particular attention to:

the overall task structure;

scene geometry and target semantics;

actions, observations, and the logic of the set_shot command;

the training split into two phases (env_cfgs_3000.py and env_cfgs_10000.py);

the rewards actually used in the pre-kick phase and in the kick phase;

terminations, PPO hyperparameters, and operational notes.

The operational reference is provided by the task/configuration files and the two reward snapshots:

task.py

rl_cfg.py

env_cfgs_3000.py

env_cfgs_10000.py

mdp.py

# 1. Expert objective

The expert models a right-footed penalty shot from a static start, with an intentionally progressive structure:

Pre-kick / biomechanical setup

The robot learns to position itself correctly with respect to the ball, keep the left support foot close to the ball, prepare the right foot, and strike in the desired direction without immediately requiring a high, highlight-style shot.

Kick / action refinement

Once the robot has learned the correct preparation, training moves to a second phase in which the robot is asked to:

strike more cleanly and repeatably;

maintain left-foot support even after impact;

give the ball a consistent launch angle, lift, and trajectory toward a high target.

This split into two training stages dramatically reduces optimization difficulty: if the policy had to learn setup + impact + lift + final precision all at once, it would likely converge too early to easy but incorrect solutions (for example: low central shots, dirty contact, losing left-foot support, or excessive crouching).

# 2. Overall pipeline structure

The pipeline has two levels.

## 2.1 Stage-1 motor decoder

The policy action does not command the 23 actuators directly in raw joint space.

The task uses MotorLatentActionCfg, so the policy operates in a latent space decoded by an external Stage-1 controller, referenced through:

environment variable MJLAB_STAGE1_WANDB_RUN_PATH_PENALTY;

stage1_wandb_run_path=os.environ.get("MJLAB_STAGE1_WANDB_RUN_PATH_PENALTY").

In practice:

Stage-1 provides a reusable motor decoder;

Stage-2 (this expert) trains the PPO policy on the penalty-shot behavior itself.

## 2.2 Stage-2 PPO expert

The registered task is:

Mjlab-Penalty-Booster-T1_23

and uses a PPO configuration with:

actor and critic MLPs (512, 256, 128), ELU activation;

entropy_coef = 0.02;

num_steps_per_env = 24;

max_iterations = 30000.

# 3. Scene geometry and coordinates

The scene uses the frame convention already consistent with the other RoboCup tasks.

## 3.1 Goal line and visual asset

It is important to distinguish between:

logical goal line: GOAL_X_LINE = 7.0

goal mesh / visual asset: GOALPOST_X = 7.3

Therefore:

the ball is evaluated as a goal when it crosses x >= 7.0 inside the valid opening;

the visual goal mesh is placed slightly farther forward, at x = 7.3.

## 3.2 Valid goal opening

The valid goal window is:

|y| <= 1.55

0 <= z <= 1.85

## 3.3 Initial ball and robot positions

Common data for both stages:

penalty distance from the goal line: 2.5 m

nominal ball position: (x=4.5, y=0.0, z=0.11)

nominal robot position: (x=4.12, y=0.04, z=default_root_height)

jitter:

ball x: +/- 0.01

robot x: +/- 0.01

robot y: +/- 0.01

yaw: +/- 0.03

## 3.4 Working area

The task uses:

a test field with half-size 7.0 x 4.5;

a goal opening consistent with goal_opening_half_width = 1.55;

a ball_out termination when the ball leaves the useful field area;

a hard_outside_area termination if the robot leaves the hard striker area.

# 4. `set_shot` command term

The core of the task is the SetShotCommand command.

## 4.1 What it contains

The command defines:

ball position;

shot target on the goal;

robot reset;

target-side sampling mode;

the possibility of entering directly into kick-only reset.

## 4.2 Target on the goal line

In both stages the target is defined directly on the goal line:

x = GOAL_X_LINE

y = visual_left_corner_y or visual_right_corner_y

z = AIM_Z

This choice is important because it keeps the following coherent:

robot orientation;

final rewards (lateral_goal, underbar_goal, goal_target_from_command);

any debug visualization marker of the target.

## 4.3 Side randomization

Both snapshots use:

TARGET_MODE = "random_binary"

So the lateral target is sampled at reset between:

VISUAL_LEFT_CORNER_Y = +1.0

VISUAL_RIGHT_CORNER_Y = -1.0

This makes it possible to generalize the movement to both corners without changing the core right-foot biomechanics of the shot.

## 4.4 Two reset modes useful for learning

The command also supports kick_only_reset_prob = 0.6.

Practical interpretation:

in 40% of resets the policy starts from the full pipeline (approach/pre-kick -> strike);

in 60% of resets it starts directly from a kick-ready pose, which speeds up learning for the impact phase.

This is a key component of the “pre-kick vs kick” split: the same task trains both the full preparation and a shorter sub-problem focused on the strike itself.

# 5. Actions and observations

# 5.1 Action

The task uses a single action:

motor_latent

The policy therefore emits a latent command decoded by the Stage-1 motor controller.

# 5.2 Actor/critic observations

Actor and critic use essentially the same set of observations.

## Robot base

base_lin_vel

base_ang_vel

projected_gravity

joint_pos

joint_vel

decoded_actions

## Penalty-task-specific information

target_dir_xy

ball_pos_rel_xyz

ball_vel_w_xy

kick_phase_flag

kick_only_reset_flag

yaw_error_abs

ball_dist_xy

right_foot_pos_rel_ball_xy

left_foot_pos_rel_ball_xy

left_support_latched_error_xy

Interpretation:

the policy knows where it is supposed to shoot (target_dir_xy);

it knows where the ball is relative to the body;

it knows whether it is already in kick phase or in kick-only reset;

it receives explicit features about left-foot support and right-foot position.

# 6. Why training is split into two stages

In this project, the two files env_cfgs_3000.py and env_cfgs_10000.py represent two configuration snapshots to be used as distinct training phases.

# 6.1 Train 1 - Pre-kick / direction / basic impact

Reference:

env_cfgs_3000.py

Idea:

lateral target already active;

vertical target still low (AIM_Z = 0.35);

final height reward still disabled;

focus on:

correct support-foot placement;

clean strike;

making the right foot actually contact the ball;

sending the ball laterally toward the desired side.

In other words: first teach the robot how to set up and strike well, not yet how to produce an immediate underbar shot.

# 6.2 Train 2 - Kick refinement / lift / high trajectory

Reference:

env_cfgs_10000.py

Idea:

higher vertical target (AIM_Z = 1.45);

lift and launch-angle rewards activated;

penalties for premature bounces and ground touches before the goal;

left-foot support latched and monitored after the strike;

final objective: turn the already-directed gesture into a high, clean shot.

In other words: the second training stage does not re-teach direction, but refines shot quality.

# 7. Train 1 (`env_cfgs_3000.py`) - pre-kick phase

## 7.1 General intuition

In the first training stage the robot must learn:

not to “sit down” too much;

not to touch the ball with the left foot too early;

not to step over the ball;

to build a right-footed movement with the left support foot close to the ball;

to already shoot laterally toward the target;

without yet emphasizing the final shot height.

## 7.2 Train 1 target parameters

lateral target: y = +/- 1.0 (random binary)

vertical target: z = 0.35

underbar_launch = 0.0

underbar_goal = 0.0

This makes the task much simpler: it primarily trains lateral control and strike mechanics.

## 7.3 Train 1 rewards

| Reward | Weight | Purpose |
| --- | --- | --- |
| right_foot_swing_intent_debug | +1.0 | Auxiliary signal to verify that the right foot really goes into swing. |
| right_only_strike | +2.0 | Bonus if the first real strike is performed with the right foot, without left-foot contact. |
| strike_event | +12.0 | Main event that recognizes the first robust strike (touch or clear ball departure). |
| impact_foot_speed | +8.0 | Rewards high right-foot speed at impact. |
| clean_strike | +10.0 | Favors clean impact, not just violent impact. |
| ball_speed_to_goal | +6.0 | Rewards ball speed toward the goal after the strike. |
| goal_scored | +12.0 | Main goal reward, in a soft target-aware form. |
| upright | +0.55 | Keeps the body in a usable posture. |
| low_height_soft_penalty | -1.8 | Penalizes collapse toward the ground. |
| double_knee_crouch | -1.5 | Prevents excessive crouching on both knees. |
| fallen | -4.0 | Fall penalty. |
| left_pre_touch | -1.0 | Penalizes the left foot touching the ball before the correct strike. |
| foot_over_ball | -2.0 | Prevents a foot from stepping over the ball. |
| foot_contact_switch | +0.2 | Small bonus for support switches that help unlock the movement. |
| support_plant_at_strike | +14.0 | Key reward: the left foot must be well planted next to the ball at the strike frame. |
| underbar_launch | 0.0 | Disabled in Train 1. |
| underbar_goal | 0.0 | Disabled in Train 1. |
| goal_target_from_command | +20.0 | Final combined reward on Y and Z at goal-line crossing. |
| lateral_goal | +18.0 | Final reward specifically for the lateral target. |

## 7.4 Most important Train 1 rewards to understand

### `support_plant_at_strike`

This is one of the key rewards of the entire expert.

It pays only once, at the frame in which the right foot truly strikes the ball, and requires that:

the left foot is near the desired position relative to the ball;

the left foot is on the ground;

the left foot is not touching the ball;

the right foot arrives with sufficient speed.

In practice, it forces the robot to build proper shot biomechanics instead of “shooting in any possible way.”

### `strike_event`

This reward is used to determine whether a real kick has occurred.

A noisy contact is not enough: the function uses both contact sensors and actual ball departure with minimum speed.

### `goal_target_from_command` and `lateral_goal`

These two rewards provide the final signal that links the gesture to the intended direction:

lateral_goal looks at the ball y coordinate when it crosses the goal line;

goal_target_from_command combines y and z.

In Train 1 the z height is intentionally of little importance, so the dominant signal is lateral direction.

# 8. Train 2 (`env_cfgs_10000.py`) - kick / high-shot phase

## 8.1 General intuition

Once the robot has learned to:

position itself correctly;

strike with the right foot;

stay laterally aligned with the target;

it moves to the second stage, where the required gesture becomes more refined:

stable left-foot support even after the strike;

right-foot velocity driven toward a target, not simply maximized;

straighter right knee at impact;

3D trajectory coherent with the target;

no premature bounces or ground touches;

high finishing under the bar.

## 8.2 Train 2 target parameters

lateral target: y = +/- 1.0

vertical target: z = 1.45

underbar_launch enabled

underbar_goal enabled

This completely changes the nature of the task: the robot must lift the shot.

## 8.3 Train 2 rewards

| Reward | Weight | Purpose |
| --- | --- | --- |
| strike_event | +12.0 | Robust strike event. |
| impact_foot_speed | +4.0 | Does not maximize absolute speed: it aims for a target right-foot speed. |
| ball_speed_to_left_aim_3d | +4.0 | Rewards 3D ball velocity toward the aim point after the strike. |
| goal_scored | +12.0 | Main goal reward, target-aware and not hard-gated. |
| post_strike_left_support_move | -3.0 | Penalizes the left foot if it moves too much after the strike. |
| post_strike_left_support_speed | -1.5 | Penalizes excessive left-foot speed during the post-strike lock window. |
| post_strike_left_support_lost_ground | -2.0 | Penalizes loss of left-foot ground contact after impact. |
| upright | +0.55 | Overall body stability. |
| low_height_soft_penalty | -2.2 | More severe than Train 1 against collapsing low. |
| double_knee_crouch | -2.0 | More severe than Train 1 against crouching. |
| fallen | -4.0 | Fall penalty. |
| left_pre_touch | -1.0 | Penalty for left-foot pre-touch. |
| foot_over_ball | -2.0 | Prevents the foot from stepping over the ball. |
| foot_contact_switch | +0.2 | Small bonus for support switches. |
| support_plant_at_strike | +14.0 | Still fundamental in the kick phase. |
| right_knee_straight_at_strike | +6.0 | Rewards a straighter right leg at impact. |
| bad_posture_at_strike | -6.0 | Penalizes striking from a wrong posture. |
| underbar_launch | +10.0 | Rewards an initial launch angle consistent with an underbar shot. |
| ball_power_lift | +6.5 | Rewards a mix of 3D speed, vx, and vz after the strike. |
| ball_ground_touch_before_goal | -6.0 | Penalizes the ball touching the ground before the goal. |
| ball_bounce_before_goal | -4.0 | Penalizes premature bounces. |
| underbar_goal | +8.0 | Final reward specifically on the Z coordinate at goal-line crossing. |
| goal_target_from_command | +14.0 | Full final reward on Y+Z. |
| lateral_goal | +12.0 | Final lateral reward. |

## 8.4 Most important Train 2 rewards to understand

### `impact_foot_speed` (target-speed version)

In Train 1, right-foot speed is pushed upward.

In Train 2 it changes into a target-speed version:

a desired speed is specified (target_speed=4.7);

the optimum is no longer “as fast as possible,” but “the correct and repeatable speed.”

This helps make the gesture more controlled.

### `post_strike_left_support_*`

These rewards are essential to prevent the robot from “letting go of everything” immediately after impact.

The left foot is latched in position and then:

it should not move too far;

it should not accelerate too much;

it should not lose ground contact.

These rewards are important because they tie shot quality to movement stability, not just to the instant of contact.

### `underbar_launch` and `ball_power_lift`

These two rewards introduce the “high-shot” physics:

underbar_launch favors the correct launch angle;

ball_power_lift rewards a combination of 3D speed, vx, and vz.

These are the reasons why Train 2 can be seen as the phase that “teaches the robot to lift the ball.”

### `ball_ground_touch_before_goal` and `ball_bounce_before_goal`

These penalties clean up the behavior:

a high shot should not become a dirty lob that bounces before the goal;

the ball should reach the goal with a cleaner trajectory.

# 9. Final rewards on the goal line

An important architectural point is that the shot target is placed directly on the goal line.

This makes three final rewards coherent.

## `lateral_goal`

Rewards the proximity of the ball y coordinate to the lateral target when the ball crosses the goal line.

## `underbar_goal`

Rewards the proximity of the ball z coordinate to the vertical target when the ball crosses the goal line.

## `goal_target_from_command`

Rewards the joint quality on y and z.

This choice removes the old ambiguity in which the ball on the goal line was compared to an aim point located farther forward in the world.

# 10. Main goal reward: why it was made “soft”

During the evolution of the task, the pure goal reward (goal_scored_event_reward) was progressively made less “blind” to the target.

The intended version is a soft target-aware goal reward:

scoring is still worth something;

scoring near the lateral target is worth more;

scoring near the vertical target is worth more;

scoring well on both is worth the maximum.

This solution is much better than a hard gate because it:

does not switch off the learning signal early on;

but still teaches the policy that the “correct” goal is worth more than an easy central goal.

# 11. Terminations and episode dynamics

The terminations common to both stages are:

| Termination | Meaning |
| --- | --- |
| fallen | persistent robot fall (low height / high tilt for multiple consecutive steps) |
| hard_outside_area | robot outside the hard striker area |
| ball_out | ball outside the useful field area |

Timing:

SIM_TIMESTEP_S = 0.005

CONTROL_DECIMATION = 4

RL dt = 0.02 s

EPISODE_LENGTH_S = 5.0

# 12. PPO / RL configuration

The PPO configuration is unique for this task.

| Parameter | Value |
| --- | --- |
| Actor hidden dims | (512, 256, 128) |
| Critic hidden dims | (512, 256, 128) |
| Activation | ELU |
| Learning rate | 1e-3 |
| Schedule | adaptive |
| Entropy coef | 0.02 |
| Gamma | 0.99 |
| Lambda | 0.95 |
| Num learning epochs | 5 |
| Num mini-batches | 4 |
| Num steps per env | 24 |
| Save interval | 500 |
| Max iterations | 30000 |

Interpretation:

the two stages do not change the PPO runner;

what changes is primarily the reward landscape.

# 13. Overall reading of the design

The expert is not “a single reward hack,” but a well-scaled pipeline.

## Train 1

It teaches:

posture;

left-foot support;

real strike execution;

lateral direction;

absence of major errors.

## Train 2

It teaches:

high shot;

better extension of the right leg;

post-strike stability;

fewer bounces;

finishing consistent with the 3D target.

This division makes sense because:

pre-kick is a problem of body setup and contact;

the high kick is a problem of ball dynamics and gesture refinement.

# 14. Conclusion

The Booster T1 penalty-shot expert is built as a progressively more complex task:

first the robot learns how to set up and how to strike;

then it learns how to refine the strike to obtain a high, clean shot consistent with the target.

The split into two training stages is not an implementation detail, but a strong methodological choice:

it separates the biomechanics of the movement from the final ball trajectory, improving learning stability and making the reward design easier to interpret.